Smart Department Automation:

Projector Switch Automation and

Lab Occupancy Display System

Session 2024 - 2028

Submitted by:

Mahnoor Shabbir		2024-CS-69

Minahil Mehmood		2024-CS-86

Aatika Yousuf     		2024-CS-95

Mahnoor              		2024-CS-96

Supervised by:

Syed Tehseen Ul Hasan Shah

Subject:

CSC-205L Computer Organization & Assembly Language

Department of Computer Science

University of Engineering and Technology Lahore

Lahore Pakistan

# Abstract:

This project presents a small-scale department automation system developed for Computer Organization and Assembly Language. The system contains two practical modules: Projector Servo Automation and Smart Lab Occupancy Indicator. The first module uses an Arduino Uno, push button, and servo motor to physically press the power button of a ceiling-mounted projector as in many classrooms, the projector button is difficult to reach, so students or teachers use a long stick to turn it on or off. The second module uses an ESP32 and a 16x2 I2C LCD to show whether Lab is free or occupied according to the stored department timetable. Students often enter labs to check availability, which can disturb an ongoing class and make the instructor lose tempo. Together, both modules solve real problems currently being faced in the department and demonstrate hardware interfacing, digital input/output, timing, stored-program execution, and microcontroller-based decision making in a deployable departmental context.

Table of Contents

1. Introduction:	4

2. Methodology:	4

3. System Overview:	4

4. Module 1: Projector Servo Automation	5

4.1 Introduction:	5

4.2 Components:	5

4.3 Implementation:	5

4.4 Complete Code:	6

4.5 Data Flow Diagram (DFD):	7

4.6 Flowchart:	7

4.7 Circuit Diagram:	8

5. Module 2: Smart Lab Occupancy Indicator	8

5.1 Introduction:	8

5.2 Components:	8

5.3 Implementation:	8

5.4 Lab 1 Timetable:	9

5.5 Complete Code:	10

5.6 Data Flow Diagram (DFD):	13

5.7 Flowchart:	13

5.8 Circuit Diagram:	14

6. Future Works:	14

7. Project Links:	14

8. References:	15

Table Of Figures:

Figure 1: System Overview	5

Table 1: Projector Servo Automation Components	5

Table 2: Projector Servo Automation Pin Connections	6

Figure 2: Projector Servo Automation Code	7

Figure 3: DFD of Projector Servo Automation	8

Figure 4: Flowchart of Projector Servo Automation	8

Figure 5: Circuit Diagram of Projector Servo Automation	9

Table 3: Smart Lab Occupancy Indicator Components	9

Table 4: Smart Lab Occupancy Indicator Pin Connections	10

Table 5: Lab1 Extracted Timetable Used in Code	10

Figure 6: DFD of Smart Lab Occupancy Indicator	14

Figure 7: Flowchart of Smart Lab Occupancy Indicator	14

Figure 8: Circuit Diagram of Smart Lab Occupancy Indicator	15

Table 6: Project Links	15

# 1. Introduction:

Departments often face small operational problems that can be reduced through simple automation. In classrooms, ceiling-mounted projectors are difficult to turn on or off when the remote is unavailable. In computer labs, students may not know whether a lab is free or occupied, causing repeated entry and disturbance during ongoing classes.

This project addresses both issues using microcontroller-based hardware. The first module automates the physical pressing of a projector button. The second module displays Lab1 availability outside the lab according to the timetable. The system is intentionally low-cost and modular, so each part can be deployed independently.

# 2. Methodology:

The project was divided into two modules. Each module was first designed at the circuit and logic level, then simulated, coded, and tested on hardware. The projector module uses a push button as input and a servo motor as output. When the button is pressed, the Arduino checks the input pin. If the button is pressed, the servo moves from its rest angle to the pressing angle. After a small delay, it moves back to its original position. This completes one button press.

The lab occupancy module uses time data and a stored timetable to decide what should appear on the LCD display. The ESP32 is used as the main controller. It gets the current time through Wi-Fi using NTP. The CS Department’s Lab1 timetable is stored in the code using a structure. Each timetable entry has the day, start time, end time, subject, teacher, and section. ESP32 checks the current day and time. If the current time falls inside any scheduled slot, the LCD shows that Lab1 is occupied. If no slot matches, the LCD shows that Lab1 is free and displays the next class time.

Both modules follow a simple embedded-system pattern: input is received, the microcontroller processes the stored instructions, and the output device performs the required action. This also supports the COAL perspective because the project demonstrates input/output interfacing, timing, control signals, and memory-stored program behavior.

# 3. System Overview:

Figure 1: System Overview

# 4. Module 1: Projector Servo Automation

## 4.1 Introduction:

Projectors in classrooms are usually mounted on the ceiling, which makes their power buttons hard to access. This creates inconvenience during lectures, especially when the remote is missing or not working. The aim of this module is to provide a simple and low-cost solution by using an Arduino-controlled servo motor. When the push button is pressed, the Arduino sends a signal to the servo motor. The servo arm rotates, presses the projector button, and then returns to its original position

## 4.2 Components:

Table 1: Projector Servo Automation Components

| Component | Purpose |
| --- | --- |
| Arduino Uno | Main controller of the system |
| Breadboard | Used for circuit assembly |
| Servo Motor | Physically presses the projector button |
| Push Button | Used to trigger the servo |
| Jumper Wires | Used to connect components |
| Connector Cable | Used to power and program the Arduino |
| Led | Used to indicate the successful working of servo |

## 4.3 Implementation:

The push button is connected to digital pin 13 of the Arduino. The input pin uses the internal pull-up resistor, so no external resistor is required. The servo signal wire is connected to digital pin 6. The servo starts at 90 degrees and rotates to the pressing angle when the button is pressed. After a short delay, the servo returns to its rest position.

Table 2: Projector Servo Automation Pin Connections

| Part | Arduino Pin |
| --- | --- |
| Push button one side | D13 |
| Push button other side | GND |
| Servo signal | D6 |
| Servo VCC | 5V |
| Servo GND | GND |
| Led button anode | D2 |
| Led button cathode | GND |

## 4.4 Code:

Figure 2: Projector Servo Automation Code

## 4.5 Data Flow Diagram (DFD):

Figure 3: DFD of Projector Servo Automation

## 4.6 Flowchart:

Figure 4: Flowchart of Projector Servo Automation

## 4.7 Circuit Diagram:

Figure 5: Circuit Diagram of Projector Servo Automation

# 5. Module 2: Smart Lab Occupancy Indicator

## 5.1 Introduction:

Students often do not know whether a lab is currently free or in use. Entering the lab to check availability can disturb an ongoing class. The Smart Lab Occupancy Indicator solves this by showing the lab status outside the lab using a small display.

## 5.2 Components:

Table 3: Smart Lab Occupancy Indicator Components

| Component | Purpose |
| --- | --- |
| ESP32 | Main controller |
| 16x2 I2C LCD | Displays lab status |
| Jumper Wires | Used for connections |
| USB Cable | Used for programming and power |

## 5.3 Implementation:

The LCD is connected to the ESP32 using I2C communication. GPIO 21 is used as SDA and GPIO 22 is used as SCL. The ESP32 gets the current time through Wi-Fi using NTP. The timetable is stored directly in the program using a structure, where every class slot has the day, start time, end time, subject, teacher, and section.

Table 4: Smart Lab Occupancy Indicator Pin Connections

| LCD Pin | ESP32 Pin |
| --- | --- |
| VCC | VIN / 5V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

The time is converted into minutes so that comparison becomes easy. For example, 08:00 is stored as 480 minutes and 13:00 is stored as 780 minutes. If the current time falls between a class start and end time, the lab is shown as occupied. Otherwise, it is shown as free and the next class is displayed.

## 5.4 Lab 1 Timetable:

Table 5: Lab1 Extracted Timetable Used in Code

| Day | Time | Status | Class / Section | Lecture / Lab | Instructor |
| --- | --- | --- | --- | --- | --- |
| Monday | 08:00-11:00 | Occupied | 4th Semester / D | ISL | Mr. Kamran Mustafa |
| Monday | 11:00-12:00 | Occupied | 4th Semester / D | DAA | Dr. Maimoona Salam |
| Monday | 13:00-16:00 | Occupied | 4th Semester / D | ADMSL | Mr. Waleed |
| Monday | 16:00-19:00 | Occupied | 2nd Semester / D | DSL | Dr. Maimoona Salam |
| Tuesday | 08:00-11:00 | Occupied | 2nd Semester / B | DSL | Ms. Hafsa |
| Tuesday | 13:00-16:00 | Occupied | 2nd Semester / D | OOPL | Mr. Muzaffar Ali |
| Tuesday | 16:00-19:00 | Occupied | 1st Semester / A | PFL | Mr. Laeeq |
| Wednesday | 08:00-09:00 | Occupied | 4th Semester / B | ToA | Dr. Tauqir Ahmed |
| Wednesday | 09:00-12:00 | Occupied | 4th Semester / B | COALL | Syed Tehseen ul Hassan Shah |
| Wednesday | 13:00-16:00 | Occupied | 2nd Semester / E | DSL | Mr. Nazeef-ul-haq |
| Thursday | 08:00-11:00 | Occupied | 4th Semester / B | ADMSL | Mr. Waleed |
| Thursday | 11:00-12:00 | Occupied | 4th Semester / B | DAA | Dr. Amna Zafar |
| Thursday | 13:00-16:00 | Occupied | 4th Semester / B | ISL | Mr. Kamran Mustafa |
| Thursday | 16:00-19:00 | Occupied | 2nd Semester / D | OOPL | Ms. Nimra Akram |
| Friday | 08:00-11:00 | Occupied | 2nd Semester / E | OOPL | Ms. Nimra Akram |

## 5.5 Code:

## 5.6 Data Flow Diagram (DFD):

Figure 6: DFD of Smart Lab Occupancy Indicator

## 5.7 Flowchart:

Figure 7: Flowchart of Smart Lab Occupancy Indicator

## 5.8 Circuit Diagram:

Figure 8: Circuit Diagram of Smart Lab Occupancy Indicator

# 6. Future Works:

The projector this module can be improved by adding a proper mounting frame for the servo motor. An external power supply can also be used for better servo stability. The system can also be upgraded by using an IR transmitter or wireless control so that the projector can be controlled without physically pressing the button. A feedback mechanism can also be added to confirm whether the projector button was pressed successfully or not.

The lab occupancy module can be extended by adding Firebase or a web dashboard so that the timetable can be updated without re-uploading the ESP32 code. Multiple labs can be supported by using one display per lab or by using a central dashboard. An RTC module can also be added to keep time when Wi-Fi is not available.

# 7. Project Links:

Table 6 Project Links

| Link Type | URL / Status |
| --- | --- |
| GitHub Repository | https://github.com/aatikayousuf/Department-Automation-Coal-Project.git |
| Wokwi Simulation - Projector Servo Automation | https://wokwi.com/projects/462899679951199233 |
| Wokwi Simulation - Lab Occupancy Indicator | https://wokwi.com/projects/new/esp32 |
| LinkedIn Post |  |

# 8. References:

- Arduino Uno Rev3 Documentation: https://docs.arduino.cc/hardware/uno-rev3/

- Arduino Servo Motor Basics: https://docs.arduino.cc/learn/electronics/servo-motors

- Arduino Digital Input Pull-Up Concept: https://docs.arduino.cc/tutorials/generic/digital-input-pullup/

- Arduino IDE Download: https://www.arduino.cc/en/software

- Wokwi Simulation Platform: https://wokwi.com/

- ESP32 Arduino Core Documentation: https://docs.espressif.com/projects/arduino-esp32/en/latest/

- ESP32 Wi-Fi API: https://docs.espressif.com/projects/arduino-esp32/en/latest/api/wifi.html

- Department Timetable: TimeTable 24-03-2026 onwards.pdf